| S/N | Sensor Name | Signal Type | Number | Location of Sensors on System | Input Voltage @ Amp | Output Signal Range | Connector and Cable Requirement | DAQ | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Accelerometer | Analog | 9 | - Gear. - Induction Motor. - Bearing x5. - On Table x2. | 18-30VDC @ 0.5mA to 8mA | 100mv/g | M12 Euro fast Connector Cable | Required |  |
| 2. | Acoustic |  | 7 | - Induction Motor. - Gear. - Bearing x5. |  |  |  | Required |  |
| 3. | Temperature sensor | Analog | 4 | - Induction Motor. - Gear. - Small Tank. - Large Tank. | 4 to 30V DC @1.5mA | 0 to 5 VDC | 3 pin Connector | Required |  |
| 4. | Voltage Sensor | Analog | 2 | - Induction Motor. - Power Supply. | 24VDC @4-20mA | 0 to 1000V AC | Not Required | Required |  |
| 5. | Current Sensor | Analog | 2 | - Induction Motor. - Power Supply. | 0 to 30A AC | 0 to 1V AC | 2 pin Terminal Block to female RCA Connector | Required |  |
| 6. | Tachometer | Digital | 2 | - Before Gear. - After Gear. | 100-240V AC @ 10VA max | 30V DC @ 100mA | Not Required | Required |  |
| 7. | Water Level Sensor (Float Switch Sensor) | Digital | 2 | - Small Tank. - Large Tank. | 220V DC@1.5A | 0 to 5V Digital Signal | Not Required | Required |  |
| 8. | Flow Sensor | Digital | 2 | - Small Tank. - Large Tank. | 5V to 15V DC @ 15mA | 0 to 5V Digital Signal | 3 pin Male Connector | Required |  |
| 9. | Humidity Sensor | Digital | 2 | - Small Tank. - Large Tank | 3.3V to 6V DC @ 1 to 1.5mA | 0 to 5V Digital Signal | 4 pin Connector |  |  |

Industrial Test Rig Simulator Sensor Description